Maximum Likelihood Estimation and Model Selection for Stationary Renewable Energy Output Data

Project Report

Vihaan Gandhi & Jash Visaria

Roll Nos: E002 & E003

Date: 20th October, 2026

# Abstract

Output from solar plants is variable, strongly periodic and contains many exact zeros, so standard statistical tools cannot be applied to it directly. This study tests which probability models and time-series models describe PV output best, using maximum likelihood estimation (MLE) and information criteria (AIC and BIC). The data are the 10-minute output of 137 Alabama PV plants for 2006 (NREL Solar Power Data for Integration Studies, as redistributed in the LSTNet repository), aggregated to 8,760 hourly values. Because 46.5% of hours are night-time zeros, the analysis works on a clear-sky index (output divided by an empirical clear-sky envelope) on daylight hours (n = 3,846). ADF and KPSS tests show that raw output is not stationary, while the index is stationary once an annual harmonic is removed. MLE was implemented from explicit log-likelihoods for seven distributions and validated against SciPy, and ARMA and GARCH models were fitted by exact maximum likelihood. The data are NREL-modelled (simulated) output, and the timestamps are reconstructed from the file layout.

A two-component normal mixture is the best distribution by both AIC and BIC (log-likelihood 1001; ΔBIC to the runner-up = 1,793), and the best single family is the scaled Beta. This ranking holds in all eight sensitivity checks. All bootstrap goodness-of-fit tests nevertheless reject every model, which is expected with strong serial dependence. A seasonal ARMA model ARMA(2,1)×(1,0,0)₂₄ captures most of the dependence (log-likelihood 6216 versus about 1,001 for the best i.i.d. model) and cuts one-hour-ahead RMSE by 13.4% relative to persistence of the index. However, all residual diagnostics for ARMA and GARCH models reject, and AIC and BIC disagree on the annual cycle. The conclusion is that the mixture and ARMA models are useful descriptions and predictors, but none of them is a correct generating model.

# 1. Introduction

Solar generation depends on the sun, the season and the weather. Planning reserves, storage and grid operation needs a quantitative description of this variability. Two descriptions are useful. A marginal distribution says how likely each output level is, regardless of when it occurs. A dynamic model says how the output at one hour depends on the preceding hours. Maximum likelihood estimation is the standard method for fitting both, and AIC and BIC give a principled way to choose between competing models.

Renewable-energy data do not automatically satisfy the assumptions behind these tools. Night-time output is exactly zero, the series has deterministic daily and annual cycles, and neighbouring hours are highly correlated. A large part of this project therefore consists of testing these assumptions, transforming the data until the methods apply, and reporting honestly where they still do not.

The study covers the following steps:

- Stationarity testing: ADF and KPSS tests, rolling statistics and group tests (hour of day, month) on raw output and on the clear-sky index.

- Maximum likelihood estimation: Normal, Student-t, Log-normal, Gamma, Weibull, scaled Beta and a two-component normal mixture, with convergence checks and validation.

- Model selection: AIC, BIC, Akaike weights and parametric-bootstrap Kolmogorov-Smirnov and Anderson-Darling tests that re-estimate the parameters.

- Time-series models: ARMA and seasonal ARMA by exact Gaussian likelihood, GARCH(1,1) on the innovations, and Ljung-Box, ARCH-LM and Jarque-Bera diagnostics.

- Forecasting: a chronological one-hour-ahead test with order selection on training data only.

The research questions are: (i) Is raw PV output stationary, and if not, what restores stationarity? (ii) Which distribution is best according to AIC and BIC, and is the ranking robust? (iii) Does any candidate pass a goodness-of-fit test when its parameters are estimated? (iv) Do ARMA and GARCH models pass residual diagnostics? (v) Does the selected dynamic model improve forecasts over simple baselines?

# 2. Methodology

## 2.1 Data Collection

The data are the Alabama solar power data for 2006 from the U.S. National Renewable Energy Laboratory (NREL) Solar Power Data for Integration Studies. The copy used is the "solar-energy" file of the public LSTNet repository (Lai et al., 2018), downloaded on 2026-10-19 (solar_AL.txt.gz; 8,748,993 bytes; SHA-256 1ae9329c…20489a1c). Each column is one plant and each row one 10-minute step. The analysis variable is the fleet total (sum over the 137 plants) averaged to hourly values.

Table 1: Dataset description

| Item | Detail |
| --- | --- |
| Source | NREL Solar Power Data for Integration Studies (Alabama, 2006), via the LSTNet repository |
| Size / frequency | 52,560 rows × 137 plants; 10-minute steps (365 × 144 rows) |
| Period | 2006-01-01 to 2006-12-31 (reconstructed time axis) |
| Quality checks | 0 missing, 0 negative, 0 duplicates, 0 always-zero plants |
| Units / variable | MW; fleet total, averaged to 8,760 hourly values |
| Analysis series | Clear-sky index on daylight hours, n = 3,846 |
| Train / test split (forecast) | Train: to 2006-10-01 (274 days); test: 91 days (758 daylight hours) |

Two limitations of the data are important. First, to the best of our knowledge the NREL integration-study data are modelled PV output derived from satellite solar-radiation and weather data for hypothetical plants, not metered generation; the NREL page could not be opened from our environment, so this should be confirmed at https://www.nrel.gov/grid/solar-power-data.html. Second, the file has no timestamps. We assumed that row 0 is 2006-01-01 00:00 and that rows follow a regular 10-minute grid. The data support this: the mean daily profile peaks at 11:40, and the number of hours with positive output per day is 10.1 in December and 13.9 in June, which matches the astronomical day length at about 32-33°N.

## 2.2 Pre-processing and Exploratory Data Analysis

No values were missing, negative or duplicated, so no imputation was needed. Hourly fleet output has mean 870 MW, standard deviation 1198 MW and maximum 4190 MW (Figure 1). Its median is only 5.2 MW because 4,076 hours (46.5%) are exactly zero; the estimated probability of zero is 0.465 (SE 0.005). These zeros are structural (night), not errors. A continuous density cannot assign probability to a single point, so the raw data are kept untouched but distribution fitting and time-series modelling use the clear-sky index described below.

Figure 1: Hourly fleet output in 2006: full year, a winter week and a summer week

Clear-sky index. For every hour of the year, the envelope E_t is the 95th percentile of output at the same hour of day within ±7 days (circular in the year). The index is

CSI_t = P_t / E_t,   for hours where E_t ≥ 10% of its annual maximum      (1)

This keeps 3,848 daylight hours. 2 of them have exactly zero output and were dropped (several candidate densities have no mass at 0), leaving n = 3,846. The index has mean 0.764, SD 0.241, skewness -1.12 and median 0.846. 233 values exceed 1 (maximum 1.25), which is natural for a 95th-percentile envelope. The monthly mean index ranges from 0.62 (February) to 0.88 (June), and its SD is about 0.29-0.30 in winter but 0.11-0.15 in summer, so weather variability is larger in the cool season. For the forecasting experiment a causal envelope (trailing 14 days) is used so that no future information enters the index.

Figure 2: Clear-sky envelope and the resulting clear-sky index

## 2.3 Stationarity Testing

A series is weakly stationary if its mean and variance are constant and its autocovariance depends only on the lag. Two tests were used at the 5% level. ADF (Dickey and Fuller, 1979): H0 = unit root (non-stationary), H1 = no unit root; a small p-value supports stationarity. KPSS (Kwiatkowski et al., 1992): H0 = level-stationary, H1 = unit root; a small p-value is evidence against stationarity. If ADF rejects and KPSS does not, the series is stationary. If both reject, there is no unit root but the mean is not constant, typically because of a deterministic cycle. Hour-of-day and month ANOVA, Kruskal-Wallis and Levene tests were also run to test directly for periodic changes in mean and variance.

Table 2: Stationarity tests (CSI = clear-sky index; KPSS p-values are limited to [0.01, 0.10])

| Series | ADF stat | ADF p | KPSS stat | KPSS p | Conclusion |
| --- | --- | --- | --- | --- | --- |
| Raw hourly output (MW) | -10.67 | < 0.001 | 2.471 | 0.010 | No unit root, mean not constant |
| Raw daily mean output (MW) | -2.85 | 0.052 | 0.823 | 0.010 | Non-stationary |
| Raw hourly, seasonal difference lag 24 | -19.08 | < 0.001 | 0.023 | 0.100 | Stationary |
| CSI, daylight hours (concatenated) | -12.40 | < 0.001 | 0.831 | 0.010 | No unit root, mean not constant |
| CSI at 12:00 (daily) | -6.27 | < 0.001 | 0.600 | 0.023 | No unit root, mean not constant |
| Daily mean clear-sky index | -7.15 | < 0.001 | 0.615 | 0.021 | No unit root, mean not constant |
| CSI minus annual harmonic | -13.58 | < 0.001 | 0.026 | 0.100 | Stationary |

Raw hourly output has no unit root (ADF p < 0.001) but is strongly non-stationary in the mean (KPSS statistic 2.47), because of the deterministic daily and annual cycles. Hour-of-day ANOVA (F = 1970) and Levene's test (267) confirm that both the mean and the variance change within the day. Seasonal differencing at lag 24 restores stationarity, which shows that the daily cycle is the main cause. The clear-sky index removes the daily pattern (index vs hour of day: ANOVA p = 0.37, Levene p = 0.49), but it still varies with the month (both p < 0.001). After removing one annual harmonic (R² = 0.100) it passes both ADF and KPSS. Its variance still differs between months, so it is stationary in the mean but not strictly in the variance. The autocorrelation is very high (lag 1: 0.950; lag 24: 0.348; Figure 3), but day-to-day persistence is weaker: the lag-1 autocorrelation of daily means is 0.38, which gives only about 165 effectively independent days.

Figure 3: ACF and PACF of the clear-sky index (pairwise-complete lags; PACF shown to lag 6 where the estimate is reliable)

## 2.4 Models Applied

### 2.4.1 Maximum likelihood estimation

For independent observations x₁,…,xₙ from a density f(x; θ), the log-likelihood is ℓ(θ) = Σ ln f(xᵢ; θ) and the maximum likelihood estimator is θ̂ = argmax ℓ(θ). Under regularity conditions θ̂ is approximately normal with covariance equal to the inverse observed information:

θ̂ ≈ N(θ, I(θ̂)⁻¹),   I(θ̂) = −∂²ℓ/∂θ∂θᵀ evaluated at θ̂      (2)

Standard errors are the square roots of its diagonal and 95% intervals are θ̂ ± 1.96·SE. Each log-likelihood was coded explicitly, parameters were mapped to unconstrained coordinates (log for positive parameters, logit for weights), and ℓ was maximised numerically with BFGS. A fit was accepted as converged only if the optimiser reported success, the gradient norm was small and the numerical Hessian of −ℓ was positive definite. Closed-form results (Normal, log-normal and the Gamma profile equation) were checked, and all fits were compared with scipy.stats.

### 2.4.2 Candidate distributions

Table 3: Candidate distributions fitted to the clear-sky index

| Model | Density | Parameters | k |
| --- | --- | --- | --- |
| Normal | (2πσ²)^(-1/2) exp(−(x−μ)²/2σ²) | μ, σ | 2 |
| Student-t | Γ((ν+1)/2) / (Γ(ν/2)√(νπ)σ) · (1+z²/ν)^(−(ν+1)/2) | μ, σ, ν | 3 |
| Log-normal | 1/(xσ√2π) exp(−(ln x − μ)²/2σ²) | μ, σ | 2 |
| Gamma | x^(a−1) e^(−x/s) / (Γ(a) s^a) | shape a, scale s | 2 |
| Weibull | (c/λ)(x/λ)^(c−1) exp(−(x/λ)^c) | shape c, scale λ | 2 |
| Scaled Beta on (0, c) | (x/c)^(α−1)(1−x/c)^(β−1) / (c·B(α,β)) | α, β (c = 1.05·max) | 2 |
| Normal mixture (2 comp.) | w·N(μ₁,σ₁²) + (1−w)·N(μ₂,σ₂²) | w, μ₁, σ₁, μ₂, σ₂ | 5 |

The Beta is fitted to x/c with the Jacobian term −n ln c, so that its log-likelihood is comparable with the others; c is fixed and not estimated. The mixture likelihood is not concave, so it was initialised by the EM algorithm (Dempster et al., 1977) from several starting points and then polished with BFGS, with a lower bound of 10⁻⁴ on each σ. The mixture represents two weather states, a broad "variable sky" component and a narrow "clear sky" component; these labels are an interpretation of the fitted components.

### 2.4.3 ARMA and GARCH models

The clear-sky index is modelled as a regression with seasonal ARMA errors on the hourly grid, with night hours treated as missing values:

φ(B) Φ(B²⁴) (Y_t − μ_t) = θ(B) ε_t,   ε_t ~ N(0, σ²),   μ_t = c (+ a·cos(2πd/365) + b·sin(2πd/365))      (3)

The exact Gaussian likelihood is computed by the Kalman filter, which handles the missing night values. It uses the same n = 3,846 observations as the i.i.d. fits, so that log-likelihoods, AIC and BIC are directly comparable. GARCH(1,1) (Engle, 1982; Bollerslev, 1986) lets the innovation variance evolve, σ_t² = ω + α ε²_{t−1} + β σ²_{t−1}, and was fitted to the ARMA innovations with normal and Student-t errors.

## 2.5 Model Selection and Evaluation

AIC = −2ℓ(θ̂) + 2k,    BIC = −2ℓ(θ̂) + k ln n      (4)

AIC (Akaike, 1974) targets predictive accuracy and BIC (Schwarz, 1978) penalises complexity more heavily for large n. Differences ΔAIC and ΔBIC to the best model and Akaike weights are reported. Information criteria rank models but do not show that the best model fits, so goodness of fit was also tested. Standard Kolmogorov-Smirnov (KS) and Anderson-Darling (AD) critical values are invalid when parameters are estimated from the same data (Lilliefors, 1967), so the null distribution was obtained by a parametric bootstrap with re-estimation in each replicate (B = 200; smallest attainable p = 1/201 = 0.005). The robustness of the ranking was checked by changing the envelope quantile (0.90, 0.98), the daylight threshold (5%, 20%), the envelope window (±15 days), and by using noon values only (n = 365) and every third day, which removes most of the dependence. Residuals were checked with the Ljung-Box test (Ljung and Box, 1978) on residuals and squared residuals, the ARCH-LM test and the Jarque-Bera test. Only the forecasting experiment uses held-out data: a chronological 75/25 train/test split of the days (Section 3.5), while the distribution and ARMA/GARCH comparisons use the whole year. The model order is chosen by BIC on the training data only, parameters are then applied with the filter so that each forecast uses data up to the previous hour, and forecasts are scored by RMSE and MAE in index units and MW against persistence baselines. All code is in the project repository (https://github.com/vihaang123/Maximum-likelihood-estimation-renewable-energy); the random seed is fixed in src/config.py. The software used was Python 3.13.16, NumPy 2.5.4, pandas 3.0.6, SciPy 1.18.1, statsmodels 0.15.0 and arch 8.0.0.

# 3. Results

## 3.1 Maximum Likelihood Estimates

All seven models converged (positive-definite Hessian) and the log-likelihoods agree with SciPy to within 10⁻⁴ or better (largest parameter difference 4.5e-05). For the mixture, SciPy has no fitter, so only its log-likelihood value was recomputed independently. Table 4 lists the estimates. The standard errors assume independent observations and are therefore too small (Section 2.3).

Table 4: Maximum likelihood estimates (clear-sky index, n = 3,846)

| Model | Parameter | Estimate (SE) | 95% CI |
| --- | --- | --- | --- |
| Normal | mu | 0.7638 (0.0039) | [0.7561, 0.7714] |
| Normal | sigma | 0.2414 (0.0028) | [0.2360, 0.2468] |
| Student-t | mu | 0.8128 (0.0058) | [0.8014, 0.8242] |
| Student-t | sigma | 0.1884 (0.0058) | [0.1770, 0.1998] |
| Student-t | nu | 4.3930 (0.5262) | [3.3617, 5.4244] |
| Log-normal | mu_log | -0.3571 (0.0081) | [-0.3729, -0.3412] |
| Log-normal | sigma_log | 0.5015 (0.0057) | [0.4903, 0.5127] |
| Gamma | shape | 5.8709 (0.1303) | [5.6156, 6.1262] |
| Gamma | scale | 0.1301 (0.0030) | [0.1242, 0.1360] |
| Weibull | shape | 3.8502 (0.0552) | [3.7421, 3.9583] |
| Weibull | scale | 0.8416 (0.0036) | [0.8345, 0.8488] |
| Beta (scaled) | alpha | 3.5597 (0.0793) | [3.4044, 3.7151] |
| Beta (scaled) | beta | 2.7184 (0.0592) | [2.6023, 2.8345] |
| 2-component normal mixture | weight1 | 0.4839 (0.0146) | [0.4553, 0.5125] |
| 2-component normal mixture | mu1 | 0.5862 (0.0087) | [0.5692, 0.6033] |
| 2-component normal mixture | sigma1 | 0.2344 (0.0043) | [0.2261, 0.2428] |
| 2-component normal mixture | mu2 | 0.9302 (0.0025) | [0.9253, 0.9352] |
| 2-component normal mixture | sigma2 | 0.0643 (0.0021) | [0.0601, 0.0684] |

The mixture puts weight 0.484 on a broad component (mean 0.586, SD 0.234) and weight 0.516 on a narrow component close to the envelope (mean 0.930, SD 0.064). Figure 4 shows that the data are clearly bimodal, which no unimodal family can reproduce.

Figure 4: Fitted densities over the histogram of the clear-sky index

## 3.2 Model Selection and Goodness of Fit

Table 5: Distribution comparison and bootstrap goodness of fit (n = 3,846)

| Model | k | Log-lik | AIC | BIC | ΔBIC | KS D | Boot. p |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2-component normal mixture | 5 | 1001.0 | -1992.1 | -1960.8 | 0.0 | 0.036 | 0.005 |
| Beta (scaled) | 2 | 92.0 | -179.9 | -167.4 | 1793.4 | 0.146 | 0.005 |
| Student-t | 3 | 45.1 | -84.3 | -65.5 | 1895.3 | 0.142 | 0.005 |
| Normal | 2 | 8.8 | -13.6 | -1.1 | 1959.8 | 0.143 | 0.005 |
| Weibull | 2 | -22.1 | 48.1 | 60.6 | 2021.5 | 0.146 | 0.005 |
| Gamma | 2 | -789.3 | 1582.5 | 1595.0 | 3555.8 | 0.185 | 0.005 |
| Log-normal | 2 | -1429.9 | 2863.9 | 2876.4 | 4837.2 | 0.207 | 0.005 |

The mixture is best by AIC and BIC, with ΔBIC = 1,793 to the scaled Beta, which is the best single family. The order of the single families is Beta, Student-t, Normal, Weibull, Gamma, Log-normal; the log-normal and Gamma are poor because they are right-skewed whereas the index is left-skewed (skewness −1.12). Every model is rejected by the bootstrap KS and AD tests (p = 0.005, the smallest value attainable with B = 200). The KS distance of the mixture (0.036) is far smaller than for the others (0.14-0.21), so it is a much better description, but not an exact one. With n = 3,846 the tests detect very small departures, and the serial dependence means that the i.i.d. bootstrap null does not reproduce the data, so no model is claimed to fit adequately.

Table 6: Sensitivity of the ranking to pre-processing choices

| Variant | n | Best by AIC and BIC | Best single family | ΔBIC to runner-up |
| --- | --- | --- | --- | --- |
| Baseline (q=0.95, +/-7 d, threshold 10%) | 3846 | Mixture | Beta (scaled) | 1793 |
| Envelope quantile 0.90 | 3830 | Mixture | Student-t | 1872 |
| Envelope quantile 0.98 | 3855 | Mixture | Beta (scaled) | 1081 |
| Daylight threshold 5% | 3973 | Mixture | Beta (scaled) | 1844 |
| Daylight threshold 20% | 3483 | Mixture | Beta (scaled) | 1699 |
| Envelope window +/-15 days | 3878 | Mixture | Beta (scaled) | 1333 |
| Noon (12:00) values only, n~365 | 365 | Mixture | Beta (scaled) | 89 |
| Every 3rd day, all daylight hours | 1284 | Mixture | Beta (scaled) | 278 |

The mixture ranks first by both criteria in all eight variants, including noon-only values and every third day, which remove most of the serial dependence. The best single family is Beta in seven variants and Student-t when the envelope quantile is lowered to 0.90.

## 3.3 Time-Series Models

Table 7: ARMA-family models (n = 3,846; same observations as the i.i.d. fits)

| Model | k | Log-lik | AIC | BIC | ΔBIC |
| --- | --- | --- | --- | --- | --- |
| ARMA(2,1)x(1,0,0)24, no annual cycle | 6 | 6216.4 | -12420.9 | -12383.3 | 0.0 |
| ARMA(2,1)x(1,0,0)24 | 8 | 6221.7 | -12427.4 | -12377.3 | 6.0 |
| ARMA(1,1)x(1,0,0)24 | 7 | 6119.0 | -12224.0 | -12180.2 | 203.2 |
| ARMA(2,1) | 7 | 5878.0 | -11742.1 | -11698.3 | 685.0 |
| AR(2) | 6 | 5849.9 | -11687.7 | -11650.2 | 733.2 |
| ARMA(1,1) | 6 | 5821.8 | -11631.6 | -11594.1 | 789.3 |
| AR(1) | 5 | 5346.6 | -10683.1 | -10651.8 | 1731.5 |
| iid Gaussian + annual cycle | 4 | 210.6 | -413.3 | -388.3 | 11995.1 |

Dependence dominates: the best ARMA log-likelihood (6216) is about six times the best i.i.d. value (1,001). The seasonal term at lag 24 is the most useful addition (ΔBIC of about 590 for ARMA(1,1) with it versus without it). BIC and AIC disagree: BIC selects the model without an annual cycle (ΔBIC = 6.0 in its favour), AIC selects the one with it (ΔAIC = 6.5). The likelihood-ratio test of the annual harmonic gives LR = 10.50 on 2 df, p = 0.0053, but this test treats the data as independent given the model, so the evidence for an annual cycle in the mean is modest.

Table 8: Parameters of the BIC-selected model ARMA(2,1)x(1,0,0)24, no annual cycle

| Parameter | Estimate | SE | z | CI low | CI high |
| --- | --- | --- | --- | --- | --- |
| constant | 0.7023 | 0.0291 | 24.2 | 0.6454 | 0.7593 |
| AR(1) φ₁ | 1.3966 | 0.0199 | 70.1 | 1.3576 | 1.4357 |
| AR(2) φ₂ | -0.4339 | 0.0194 | -22.3 | -0.4721 | -0.3958 |
| MA(1) θ₁ | 0.3048 | 0.0243 | 12.5 | 0.2571 | 0.3524 |
| Seasonal AR(24) Φ | 0.4165 | 0.0116 | 35.8 | 0.3937 | 0.4394 |
| σ² | 0.0016 | 0.0000 | 56.8 | 0.0015 | 0.0016 |

The estimates describe a persistent but stationary process (φ₁ + φ₂ = 0.963 < 1) with day-to-day persistence at the same hour (Φ = 0.42). For volatility, GARCH(1,1) with Student-t errors is preferred by a wide margin (log-likelihood 5897 vs 4336 for constant variance), but the fit is degenerate: α = 0.890, β = 0.110 (α + β = 1) and ν = 2.16 degrees of freedom. This suggests that the recursion is absorbing a different structure, namely level-dependent variance (Section 3.4), rather than true volatility clustering.

## 3.4 Residual Diagnostics

Table 9: Residual diagnostics (H0: no autocorrelation / no ARCH effect / normality)

| Model | Test | Statistic | p-value | Reject H0 |
| --- | --- | --- | --- | --- |
| ARMA(2,1)×(1,0,0)₂₄ | Ljung-Box (lag 24) | 120.6 | < 0.001 | Yes |
| ARMA(2,1)×(1,0,0)₂₄ | Ljung-Box (squared residuals) (lag 24) | 276.0 | < 0.001 | Yes |
| ARMA(2,1)×(1,0,0)₂₄ | ARCH-LM (lag 12) | 171.7 | < 0.001 | Yes |
| ARMA(2,1)×(1,0,0)₂₄ | Jarque-Bera | 883.3 | < 0.001 | Yes |
| GARCH(1,1)-t (standardised) | Ljung-Box (lag 24) | 80.9 | < 0.001 | Yes |
| GARCH(1,1)-t (standardised) | Ljung-Box (squared residuals) (lag 24) | 166.5 | < 0.001 | Yes |
| GARCH(1,1)-t (standardised) | ARCH-LM (lag 12) | 107.1 | < 0.001 | Yes |
| GARCH(1,1)-t (standardised) | Jarque-Bera | 80161.3 | < 0.001 | Yes |

All 10 diagnostic tests reject for every ARMA model and for the GARCH model (full results in results/diagnostic_results.csv). None of the fitted dynamic models is adequate in the strict sense. Three specific features explain this:

The night gap. The residual ACF is -0.06 at lag 1 but -0.57 at lag 13. On the hourly grid this links an hour to the one 13 hours earlier, which for early-morning hours lies on the previous afternoon, across the night. The ARMA structure on the 24-hour grid does not represent this dependence. Changing the assumed gap length (0, 4, 12 or 24 night slots) gave log-likelihoods of 4,907, 6,117, 6,222 and 6,223, so the true 12-slot gap is already near the optimum.

Level-dependent variance. The residual SD falls from 1.39 in the lowest quintile of fitted level to 0.56 in the highest: variability is large when the index is expected to be low and small when it is near 1. A logit transformation to stabilise the variance was tried, but it gave a lower log-likelihood on the original scale (6,012 vs 6,222) and was rejected.

Non-normal errors. Residual skewness is −0.58 and excess kurtosis 2.04, so Jarque-Bera rejects.

Figure 5: Residual diagnostics of the selected ARMA model

## 3.5 Out-of-Sample Forecasting

Where the train/test split is used. A chronological split was used for the forecasting experiment only. The first 75% of days form the training set and the last 25% the hold-out test set; the data are never shuffled, and no test data are used for estimation or order selection. The distribution fits (Sections 3.1 and 3.2) and the ARMA and GARCH model comparisons (Section 3.3) describe the whole year, so they are in-sample comparisons by AIC and BIC; they are not scored on held-out data.

Table 10: Chronological train/test split used for forecasting

| Set | Start | End | Days | Daylight hours | Used for |
| --- | --- | --- | --- | --- | --- |
| Training | 2006-01-01 | 2006-10-01 | 274 | 2870 | order selection (BIC) and parameter estimation |
| Test (hold-out) | 2006-10-02 | 2006-12-31 | 91 | 849 | forecast scoring (758 hours with an observed previous hour) |

Training covers the first 274 days (to 2006-10-01) and testing the last 91 days (from 2006-10-02), so the test period is October to December. The table counts daylight hours with a valid index; the fitted training sample contained 2,868 observations, and 758 test hours were scored (those whose previous hour is also observed). BIC on the training data selected ARMA(1,1)×(1,0,0)₂₄ without harmonics (Table 11).

Figure 6: Chronological train/test split of the clear-sky index (causal-envelope version used for forecasting)

Table 11: One-hour-ahead forecast accuracy on the hold-out period (skill = 1 − RMSE / RMSE of index persistence)

| Method | MAE (index) | RMSE (index) | MAE (MW) | RMSE (MW) | RMSE skill |
| --- | --- | --- | --- | --- | --- |
| ARMA(1,1)×(1,0,0)₂₄ | 0.0381 | 0.0532 | 94.0 | 127.7 | 0.134 |
| Persistence of index | 0.0456 | 0.0684 | 105.5 | 147.4 | 0.000 |
| Training-mean index | 0.2353 | 0.3035 | 631.5 | 842.5 | -4.716 |
| Naive power persistence (P_{t-1}) | – | – | 452.6 | 569.9 | -2.866 |

The ARMA model has an RMSE of 127.7 MW against 147.4 MW for persistence of the index (a gain of 13.4%). Both are much better than persistence of raw power (570 MW), which cannot anticipate sunrise or sunset. The test covers a single 91-day period (October to December) and no significance test was run, so the margin over persistence is indicative only.

Figure 7: Forecast evaluation on the hold-out period

## 3.6 Summary of Results

Table 12: Overall summary

| Question | Result |
| --- | --- |
| Stationarity | Raw output: not stationary (daily and annual cycles). Clear-sky index minus annual harmonic: stationary (ADF and KPSS). |
| Best distribution | Two-component normal mixture (BIC -1961); best single family: scaled Beta (BIC -167). Robust in 8 of 8 variants. |
| Goodness of fit | All 7 models rejected by bootstrap KS and AD (p = 0.005); mixture much closer (D = 0.036). |
| Best time-series model | ARMA(2,1)×(1,0,0)₂₄: BIC selects no annual cycle, AIC selects it (LR p = 0.0053). |
| Diagnostics | All ARMA and GARCH residual tests reject; GARCH fit is degenerate (α + β = 1). |
| Train/test split | Chronological 75/25: 274 training days, 91 test days (forecasting only; other comparisons are in-sample). |
| Forecasting | RMSE 128 MW vs 147 MW for index persistence (13.4% lower). |

# 4. Discussion

The main message is that the shape of the PV output distribution is better explained by two weather states than by any single family. On clear days the index sits close to the envelope with little spread, whereas on variable or overcast days it is spread over a wide range. A unimodal density has to compromise between the two, which is why the single families, including the Beta, fall far behind. The ranking stayed the same even when most of the serial dependence was removed (noon-only values and every third day), which supports the conclusion despite the dependence.

The comparison between i.i.d. and ARMA likelihoods should be read carefully. The two kinds of model answer different questions: the mixture describes the marginal distribution, the ARMA model the conditional distribution given the past. The gap in log-likelihood shows how much information the past contains, not that ARMA "beats" the mixture as a marginal model. A natural combination, which was not attempted, is a time-series model with Beta or mixture innovations.

Model selection also needs caution. AIC and BIC for the i.i.d. models assume independent observations. With only about 165 effectively independent days, the BIC penalty based on n = 3,846 is too light and the likelihoods overstate the information in the data. That the bootstrap tests reject every model, and every ARMA and GARCH diagnostic rejects, should be read in the same light: the models are useful approximations. The diagnostics point to specific improvements, namely dependence across the night and variance that depends on the level of the index.

Limitations. The data are modelled rather than metered output and the timestamps are reconstructed; only one year, one state and the fleet total were studied; the clear-sky envelope is empirical rather than physical, so the index depends on the envelope quantile and window; and the forecast evaluation uses a single origin and a 91-day test period.

# 5. Conclusion

This project implemented maximum likelihood estimation and model selection for solar PV output from first principles and tested it on real, public data. Five conclusions follow:

- Raw PV output is not stationary because of the daily and annual cycles and contains 46.5% structural zeros. A clear-sky index on daylight hours, minus an annual harmonic, is stationary and is the appropriate object for fitting.

- A two-component normal mixture is the best distribution by AIC and BIC in the baseline and in all eight sensitivity checks; the scaled Beta is the best single family.

- Every distribution is rejected by parametric-bootstrap goodness-of-fit tests, mainly because of serial dependence, so the best model is only the best of those compared.

- A seasonal ARMA model captures most of the dependence and reduces forecast RMSE by 13.4% relative to persistence, but it fails all residual diagnostics, as does GARCH. AIC and BIC disagree on the annual cycle.

- The mixture and ARMA models are useful descriptions and predictors, but none of them should be regarded as the true generating model.

# 6. Future Scope

Future work could (i) replace the empirical envelope with a physical clear-sky model (for example from pvlib) and use metered data when available; (ii) use a block bootstrap or models with dependent errors for valid inference and goodness-of-fit tests; (iii) build a two-time-scale model (a day-level random effect plus within-day dynamics) and allow level-dependent variance, or use Beta or mixture innovations; (iv) try regime-switching models and mixtures with more components; (v) evaluate forecasts with rolling origins and probabilistic scores such as CRPS; and (vi) apply the same pipeline to wind data, where a Weibull-based analysis is the natural starting point.

# References

Akaike, H. (1974). A new look at the statistical model identification. IEEE Transactions on Automatic Control, 19(6), 716–723.

Bollerslev, T. (1986). Generalized autoregressive conditional heteroskedasticity. Journal of Econometrics, 31(3), 307–327.

Box, G. E. P., Jenkins, G. M., Reinsel, G. C., & Ljung, G. M. (2015). Time Series Analysis: Forecasting and Control (5th ed.). Wiley.

Burnham, K. P., & Anderson, D. R. (2002). Model Selection and Multimodel Inference (2nd ed.). Springer.

Dempster, A. P., Laird, N. M., & Rubin, D. B. (1977). Maximum likelihood from incomplete data via the EM algorithm. Journal of the Royal Statistical Society, Series B, 39(1), 1–38.

Dickey, D. A., & Fuller, W. A. (1979). Distribution of the estimators for autoregressive time series with a unit root. Journal of the American Statistical Association, 74(366), 427–431.

Efron, B. (1979). Bootstrap methods: another look at the jackknife. The Annals of Statistics, 7(1), 1–26.

Engle, R. F. (1982). Autoregressive conditional heteroscedasticity with estimates of the variance of United Kingdom inflation. Econometrica, 50(4), 987–1007.

Fisher, R. A. (1922). On the mathematical foundations of theoretical statistics. Philosophical Transactions of the Royal Society A, 222, 309–368.

Hyndman, R. J., & Koehler, A. B. (2006). Another look at measures of forecast accuracy. International Journal of Forecasting, 22(4), 679–688.

Inman, R. H., Pedro, H. T. C., & Coimbra, C. F. M. (2013). Solar forecasting methods for renewable energy integration. Progress in Energy and Combustion Science, 39(6), 535–576.

Jarque, C. M., & Bera, A. K. (1987). A test for normality of observations and regression residuals. International Statistical Review, 55(2), 163–172.

Kwiatkowski, D., Phillips, P. C. B., Schmidt, P., & Shin, Y. (1992). Testing the null hypothesis of stationarity against the alternative of a unit root. Journal of Econometrics, 54(1–3), 159–178.

Lai, G., Chang, W.-C., Yang, Y., & Liu, H. (2018). Modeling long- and short-term temporal patterns with deep neural networks. Proceedings of the 41st International ACM SIGIR Conference, 95–104. Data: https://github.com/laiguokun/multivariate-time-series-data

Lilliefors, H. W. (1967). On the Kolmogorov-Smirnov test for normality with mean and variance unknown. Journal of the American Statistical Association, 62(318), 399–402.

Ljung, G. M., & Box, G. E. P. (1978). On a measure of lack of fit in time series models. Biometrika, 65(2), 297–303.

National Renewable Energy Laboratory. Solar Power Data for Integration Studies. https://www.nrel.gov/grid/solar-power-data.html

Schwarz, G. (1978). Estimating the dimension of a model. The Annals of Statistics, 6(2), 461–464.

Seabold, S., & Perktold, J. (2010). statsmodels: econometric and statistical modeling with Python. Proceedings of the 9th Python in Science Conference, 92–96.

Sheppard, K. arch: ARCH models in Python (software). https://github.com/bashtage/arch

Virtanen, P., et al. (2020). SciPy 1.0: fundamental algorithms for scientific computing in Python. Nature Methods, 17, 261–272.

# Appendix: Additional Outputs

This appendix collects every remaining figure and table produced by the project (all files are also in results/ and results/figures/ in the repository), so that the report contains the complete set of outputs.

## A. Additional figures

Figure 8: Diurnal profile and seasonal pattern of raw hourly output

Figure 9: Distribution of raw hourly output (all hours and strictly positive hours)

Figure 10: Rolling statistics and group effects (hour of day, month)

Figure 11: ACF and PACF of the raw hourly output

Figure 12: Log-likelihood surface of the Weibull model with the MLE marked

Figure 13: Q-Q plots of the clear-sky index against each fitted distribution

Figure 14: AIC and BIC comparison of the candidate distributions

Figure 15: I.i.d. distribution models versus dynamic models on the same observations

Figure 16: GARCH conditional volatility of the ARMA innovations

## B. Additional tables

Table 13: Complete ADF and KPSS results (all specifications)

| Series | Test | Statistic | p-value | 5% crit. | Reject H0 |
| --- | --- | --- | --- | --- | --- |
| Raw hourly output (MW) | ADF ('c', AIC lags) | -10.674 | < 0.001 | -2.862 | True |
| Raw hourly output (MW) | KPSS ('c', bw=auto (data-driven)) | 2.471 | 0.010 | 0.463 | True |
| Raw hourly output (MW) | KPSS ('c', bw=37 (long rule)) | 1.674 | 0.010 | 0.463 | True |
| Raw hourly output (MW) | KPSS ('ct', bw=auto (data-driven)) | 1.961 | 0.010 | 0.146 | True |
| Raw daily mean output (MW) | ADF ('c', AIC lags) | -2.847 | 0.052 | -2.870 | False |
| Raw daily mean output (MW) | KPSS ('c', bw=auto (data-driven)) | 0.823 | 0.010 | 0.463 | True |
| Raw daily mean output (MW) | KPSS ('c', bw=17 (long rule)) | 0.534 | 0.034 | 0.463 | True |
| Raw daily mean output (MW) | KPSS ('ct', bw=auto (data-driven)) | 0.670 | 0.010 | 0.146 | True |
| Raw hourly, seasonal difference lag 24 | ADF ('c', AIC lags) | -19.077 | < 0.001 | -2.862 | True |
| Raw hourly, seasonal difference lag 24 | KPSS ('c', bw=auto (data-driven)) | 0.023 | 0.100 | 0.463 | False |
| Raw hourly, seasonal difference lag 24 | KPSS ('c', bw=37 (long rule)) | 0.022 | 0.100 | 0.463 | False |
| Raw hourly, seasonal difference lag 24 | KPSS ('ct', bw=auto (data-driven)) | 0.003 | 0.100 | 0.146 | False |
| CSI, daylight hours (concatenated) | ADF ('c', AIC lags) | -12.400 | < 0.001 | -2.862 | True |
| CSI, daylight hours (concatenated) | KPSS ('c', bw=auto (data-driven)) | 0.831 | 0.010 | 0.463 | True |
| CSI, daylight hours (concatenated) | KPSS ('c', bw=30 (long rule)) | 0.872 | 0.010 | 0.463 | True |
| CSI, daylight hours (concatenated) | KPSS ('ct', bw=auto (data-driven)) | 0.712 | 0.010 | 0.146 | True |
| CSI at 12:00 (daily) | ADF ('c', AIC lags) | -6.272 | < 0.001 | -2.870 | True |
| CSI at 12:00 (daily) | KPSS ('c', bw=auto (data-driven)) | 0.600 | 0.023 | 0.463 | True |
| CSI at 12:00 (daily) | KPSS ('c', bw=17 (long rule)) | 0.379 | 0.086 | 0.463 | False |
| CSI at 12:00 (daily) | KPSS ('ct', bw=auto (data-driven)) | 0.577 | 0.010 | 0.146 | True |
| Daily mean clear-sky index | ADF ('c', AIC lags) | -7.155 | < 0.001 | -2.870 | True |
| Daily mean clear-sky index | KPSS ('c', bw=auto (data-driven)) | 0.615 | 0.021 | 0.463 | True |
| Daily mean clear-sky index | KPSS ('c', bw=17 (long rule)) | 0.402 | 0.076 | 0.463 | False |
| Daily mean clear-sky index | KPSS ('ct', bw=auto (data-driven)) | 0.549 | 0.010 | 0.146 | True |
| CSI minus annual harmonic | ADF ('c', AIC lags) | -13.585 | < 0.001 | -2.862 | True |
| CSI minus annual harmonic | KPSS ('c', bw=auto (data-driven)) | 0.026 | 0.100 | 0.463 | False |
| CSI minus annual harmonic | KPSS ('c', bw=30 (long rule)) | 0.026 | 0.100 | 0.463 | False |
| CSI minus annual harmonic | KPSS ('ct', bw=auto (data-driven)) | 0.022 | 0.100 | 0.146 | False |

Table 14: Group tests for periodic changes in mean and variance

| Series | Factor | Test | Statistic | p-value | Reject H0 |
| --- | --- | --- | --- | --- | --- |
| Raw hourly output | hour of day | One-way ANOVA | 1969.96 | < 0.001 | True |
| Raw hourly output | hour of day | Kruskal-Wallis | 8035.42 | < 0.001 | True |
| Raw hourly output | hour of day | Levene (Brown-Forsythe) | 266.53 | < 0.001 | True |
| Raw hourly output | month | One-way ANOVA | 15.64 | < 0.001 | True |
| Raw hourly output | month | Kruskal-Wallis | 149.65 | < 0.001 | True |
| Raw hourly output | month | Levene (Brown-Forsythe) | 16.04 | < 0.001 | True |
| Clear-sky index | hour of day | One-way ANOVA | 1.09 | 0.366 | False |
| Clear-sky index | hour of day | Kruskal-Wallis | 14.89 | 0.188 | False |
| Clear-sky index | hour of day | Levene (Brown-Forsythe) | 0.95 | 0.490 | False |
| Clear-sky index | month | One-way ANOVA | 42.22 | < 0.001 | True |
| Clear-sky index | month | Kruskal-Wallis | 252.90 | < 0.001 | True |
| Clear-sky index | month | Levene (Brown-Forsythe) | 55.84 | < 0.001 | True |

Table 15: Validation of the own MLE code against SciPy (for the mixture only the likelihood value is recomputed independently)

| Model | Own log-lik | SciPy log-lik | Difference | Max ¦Δθ¦ | Validated |
| --- | --- | --- | --- | --- | --- |
| Normal | 8.793 | 8.793 | -3.7e-13 | 6.4e-10 | True |
| Student-t | 45.127 | 45.127 | 2.2e-08 | 4.8e-06 | True |
| Log-normal | -1429.940 | -1429.940 | 0.0e+00 | 1.4e-09 | True |
| Gamma | -789.255 | -789.255 | 8.0e-13 | 1.8e-07 | True |
| Weibull | -22.069 | -22.069 | 4.1e-07 | 4.5e-05 | True |
| Beta (scaled) | 91.959 | 91.959 | 4.5e-12 | 7.7e-08 | True |
| 2-component normal mixture | 1001.050 | 1001.050 | 0.0e+00 | 0.0e+00 | True |

Table 16: All ARMA-family models (n = 3,846)

| Model | k | Log-lik | AIC | BIC | ΔAIC | ΔBIC |
| --- | --- | --- | --- | --- | --- | --- |
| ARMA(2,1)x(1,0,0)24, no annual cycle | 6 | 6216.4 | -12420.9 | -12383.3 | 6.5 | 0.0 |
| ARMA(2,1)x(1,0,0)24 | 8 | 6221.7 | -12427.4 | -12377.3 | 0.0 | 6.0 |
| ARMA(1,1)x(1,0,0)24 | 7 | 6119.0 | -12224.0 | -12180.2 | 203.4 | 203.2 |
| ARMA(2,1) | 7 | 5878.0 | -11742.1 | -11698.3 | 685.3 | 685.0 |
| ARMA(2,2) | 8 | 5881.1 | -11746.2 | -11696.2 | 681.2 | 687.2 |
| AR(3) | 7 | 5869.5 | -11725.0 | -11681.3 | 702.3 | 702.1 |
| AR(2) | 6 | 5849.9 | -11687.7 | -11650.2 | 739.7 | 733.2 |
| ARMA(1,1) | 6 | 5821.8 | -11631.6 | -11594.1 | 795.8 | 789.3 |
| AR(1) | 5 | 5346.6 | -10683.1 | -10651.8 | 1744.3 | 1731.5 |
| iid Gaussian + annual cycle | 4 | 210.6 | -413.3 | -388.3 | 12014.1 | 11995.1 |

Table 17: Volatility models for the ARMA innovations

| Model | k | Log-lik | AIC | BIC | ΔBIC |
| --- | --- | --- | --- | --- | --- |
| GARCH(1,1) (Student-t) | 4 | 5896.6 | -11785.2 | -11760.2 | 0.0 |
| GARCH(1,1) (normal) | 3 | 4474.4 | -8942.7 | -8923.9 | 2836.2 |
| Constant variance (normal) | 1 | 4335.8 | -8669.6 | -8663.3 | 3096.8 |
| ARCH(1) (normal) | 2 | 4335.8 | -8667.6 | -8655.1 | 3105.1 |

Table 18: Parameters of the selected GARCH(1,1)-t model

| Model | Parameter | Estimate | SE | p-value |
| --- | --- | --- | --- | --- |
| GARCH(1,1) (Student-t) | omega | 84.0105 | 17.4697 | 1.5e-06 |
| GARCH(1,1) (Student-t) | alpha[1] | 0.8897 | 0.1962 | 5.8e-06 |
| GARCH(1,1) (Student-t) | beta[1] | 0.1103 | 0.0336 | 0.001 |
| GARCH(1,1) (Student-t) | nu | 2.1596 | 0.0308 | 0 |

Table 19: Residual-test p-values for every model (LB = Ljung-Box, LB² = on squared residuals, ARCH = ARCH-LM, JB = Jarque-Bera; lag in brackets). Every test rejects at the 5% level.

| Model | LB(10) | LB(24) | LB(48) | LB²(10) | LB²(24) | LB²(48) | ARCH(5) | ARCH(12) | ARCH(24) | JB |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2-component normal mixture (i.i.d., PIT) | <0.001 | <0.001 | <0.001 | – | – | – | – | – | – | – |
| iid Gaussian + annual cycle | <0.001 | <0.001 | <0.001 | <0.001 | <0.001 | <0.001 | <0.001 | <0.001 | <0.001 | <0.001 |
| AR(1) | <0.001 | <0.001 | <0.001 | <0.001 | <0.001 | <0.001 | <0.001 | <0.001 | <0.001 | <0.001 |
| AR(2) | <0.001 | <0.001 | <0.001 | <0.001 | <0.001 | <0.001 | <0.001 | <0.001 | <0.001 | <0.001 |
| AR(3) | <0.001 | <0.001 | <0.001 | <0.001 | <0.001 | <0.001 | <0.001 | <0.001 | <0.001 | <0.001 |
| ARMA(1,1) | <0.001 | <0.001 | <0.001 | <0.001 | <0.001 | <0.001 | <0.001 | <0.001 | <0.001 | <0.001 |
| ARMA(2,1) | <0.001 | <0.001 | <0.001 | <0.001 | <0.001 | <0.001 | <0.001 | <0.001 | <0.001 | <0.001 |
| ARMA(2,2) | <0.001 | <0.001 | <0.001 | <0.001 | <0.001 | <0.001 | <0.001 | <0.001 | <0.001 | <0.001 |
| ARMA(1,1)x(1,0,0)24 | <0.001 | <0.001 | <0.001 | <0.001 | <0.001 | <0.001 | <0.001 | <0.001 | <0.001 | <0.001 |
| ARMA(2,1)x(1,0,0)24 | <0.001 | <0.001 | <0.001 | <0.001 | <0.001 | <0.001 | <0.001 | <0.001 | <0.001 | <0.001 |
| ARMA(2,1)x(1,0,0)24, no annual cycle | <0.001 | <0.001 | <0.001 | <0.001 | <0.001 | <0.001 | <0.001 | <0.001 | <0.001 | <0.001 |
| GARCH(1,1) (Student-t) (std. resid.) | <0.001 | <0.001 | <0.001 | <0.001 | <0.001 | <0.001 | 0.032 | <0.001 | <0.001 | <0.001 |

Table 20: Residual standard deviation by quintile of fitted level

| Fitted-level quintile | Residual SD | n | Mean fitted index |
| --- | --- | --- | --- |
| Q1 | 1.386 | 770 | 0.370 |
| Q2 | 1.211 | 769 | 0.688 |
| Q3 | 0.887 | 769 | 0.836 |
| Q4 | 0.705 | 769 | 0.925 |
| Q5 | 0.562 | 769 | 0.991 |

Table 21: Residual standard deviation by hour of day

| Hour of day | Residual SD | n |
| --- | --- | --- |
| 6 | 0.729 | 185 |
| 7 | 1.042 | 364 |
| 8 | 0.991 | 364 |
| 9 | 1.230 | 365 |
| 10 | 0.963 | 365 |
| 11 | 0.771 | 365 |
| 12 | 0.818 | 365 |
| 13 | 0.815 | 365 |
| 14 | 0.847 | 365 |
| 15 | 0.937 | 365 |
| 16 | 1.230 | 282 |
| 17 | 1.435 | 96 |

Table 22: Sensitivity of the ARMA fit to the assumed length of the night gap

| Night slots | Seasonal period | n | k | loglik | AIC | BIC |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | 12 | 3846 | 8 | 4906.7 | -9797.4 | -9747.4 |
| 4 | 16 | 3846 | 8 | 6117.4 | -12218.8 | -12168.8 |
| 12 | 24 | 3846 | 8 | 6221.7 | -12427.4 | -12377.3 |
| 24 | 36 | 3846 | 8 | 6223.0 | -12430.1 | -12380.0 |

Table 23: Order selection on the training sample (forecasting)

| model | n | k | Log-lik | AIC | BIC | converged |
| --- | --- | --- | --- | --- | --- | --- |
| ARMA(1,1)x(1,0,0)24 | 2868 | 5 | 4437.4 | -8864.8 | -8835.0 | True |
| ARMA(2,1) | 2868 | 5 | 4299.5 | -8589.0 | -8559.2 | True |
| AR(2) | 2868 | 4 | 4258.8 | -8509.5 | -8485.7 | True |
| ARMA(1,1) | 2868 | 4 | 4247.2 | -8486.4 | -8462.5 | True |
| AR(1) | 2868 | 3 | 3846.8 | -7687.6 | -7669.7 | True |

Table 24: Supplementary comparison on raw positive output in MW (n = 4,684)

| Model | n | k | Log-lik | converged | optimizer_success | iterations | max_abs_grad_per_obs | information_matrix_pd | implied_mean | implied_sd | quantile_rmse | AIC | BIC | AICc | delta_AIC | weight_AIC | rank_AIC | delta_BIC | weight_BIC | rank_BIC |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Beta (scaled) | 4684 | 2 | -37867.2 | True | True | 51 | 3.908e-08 | True | 1439 | 1286 | 336.8 | 7.574e+04 | 75751.2 | 7.574e+04 | 0 | 1 | 1 | 0 | 1 | 1 |
| 2-component normal mixture | 4684 | 5 | -38576.7 | True | True | 212 | 8.882e-09 | True | 1628 | 1205 | 188.8 | 7.716e+04 | 77195.7 | 7.716e+04 | 1425 | 3.497e-310 | 2 | 1444 | 2.191e-314 | 2 |
| Gamma | 4684 | 2 | -38874.3 | True | True | 11 | 3.73e-08 | True | 1628 | 2075 | 1041 | 7.775e+04 | 77765.5 | 7.775e+04 | 2014 | 0 | 3 | 2014 | 0 | 3 |
| Weibull | 4684 | 2 | -39143.0 | True | True | 7 | 2.665e-09 | True | 1717 | 2198 | 1121 | 7.829e+04 | 78302.9 | 7.829e+04 | 2552 | 0 | 4 | 2552 | 0 | 4 |
| Normal | 4684 | 2 | -39875.8 | True | True | 32 | 6.928e-08 | True | 1628 | 1205 | 281 | 7.976e+04 | 79768.6 | 7.976e+04 | 4017 | 0 | 5 | 4017 | 0 | 5 |
| Student-t | 4684 | 3 | -39875.8 | False | True | 173 | 0.008262 | True | 1628 | 1205 | 281 | 7.976e+04 | 79777.1 | 7.976e+04 | 4019 | 0 | 6 | 4026 | 0 | 6 |
| Log-normal | 4684 | 2 | -40888.2 | True | False | 2 | 2.487e-08 | True | 1.356e+04 | 3.07e+05 | 2.397e+04 | 8.178e+04 | 81793.3 | 8.178e+04 | 6042 | 0 | 7 | 6042 | 0 | 7 |

Table 25: Summary of the 137 plants (full table in results/plant_summary.csv)

| Statistic | Value |
| --- | --- |
| Mean of plant means (MW) | 6.353 |
| Smallest plant mean (MW) | 1.606 |
| Largest plant mean (MW) | 19.131 |
| Largest single-plant output (MW) | 88.900 |
| Average share of zero 10-minute values | 0.551 |